August/2025

Mohamed adel abdelrahem ahmed

Analog Devices       Eng. Omar fahmy

FFT

project

Name : Mohamed Adel Abdelrahem Ahmed 
mail : mhmdadel344@gmail.com

FFT

Introduction

This is the final project of the internship @ Analog Devices.

We are going to implement the FFT with radix-2 and N=8 with MATLAB and then follow the hardware steps (digital flow)

We will not cover the Verification and Backend steps till now.

FFT

It’s “fast fourier transform” an algorithm that computes the discrete fourier transform “DFT” of a sequence , or its inverse.

A fourier transform converts a signal from its original domain to a representation in the frequency domain and vice versa.

FFT manages to reduce the complexity of computing the DFT from O(n^2) to O(nxlog n ) where n is the data size.

The difference in speed can be enormous , especially for long data sets where n may be in the thousands or millions.

For Radix-2:

- The input size N must be a power of 2 (here, 8).

- The algorithm recursively splits the signal into even and odd-indexed parts, computes FFT of both, and combines using twiddle factors.

MATLAB Part

1st step I isolated the function file from the test files

2nd step check for codegen

We wrote the fft in a loop based and bit-reversed iterative style

3rd step

This created the mex file

4th , 5th ,6th  step

Making type table

Test

This table will be used in step 6 to cast all variables to fixed-point versions

Since the WL=12 and FL=8 then its 4-bit for integer and 8-bit for fraction

Then maximum +ve is 0111.11111111 = +7.996

And maximum -ve is    1000.00000000 = -8

I changed my code part that randomize the input to be more general using randn

Ex of simulation in matlab

the full codes will be in separate files .m

Arch design

We will choose iterative in place architecture

RTL blocks:

Flow of the design : first we inputs the 8 samples x(0) x(1) … x(7) real and img with size 12-bit to the regfile and then passed it to the butterfly inputs (U,V) and then the butterfly make the calculations to multiply the V with the W “twiddle factor” and the U multiplied by 1 and after that butterfly outputs (out1 ,out2) out1=U+V.W and out2=U-V.W , and after calculation we then rewrite the regfile with the result.

The FSM control the flow and provide the W values for the butterfly and provide the address to the regfile and tell what stage are we in now and the butterfly index.

Twiddle factor & radix-2 algorithm

After assuming WNm=e-j2pi(m/N)

We can make it 4 2-points DFT

Replacing the 2-point DFT by butterfly

RTL files will be in separate folder.

Results

First the MATLAB randomize 8-sample input and then do the FFT and generate the output and compute the AVG. SQNR and also the error between my FFT model and internal MATLAB FFT and make 2 files one for the inputs and other for the outputs to pass them to the testbench done to test also the RTL design.

The 2 generated files input , output

QUESTASIM do file

Questa result

It’s written in decimal but easily divided by 256 to see it in format Q4.8

Just like matlab

Wave form trace & latency calculation

The full wave form

Zoomed in for loading the input data

As we see here the blue signals is the input/output file from matlab so after the load signal is high we begin to take the inputs in 8 cycles so after 8 cycles from load the orange signal is the internal memory in the design that hold the inputs data.

After that we have start signal it goes 1 when loading all input and then the calculation starts and after all calculations end then done signal go high.

We need to calculate the latency “number of cycles from start of the calculations to the first output”. If we need execution time it will be “number of cycles from start of the calculations to the done=1”.

As we see from the start to the first data_out_re , data_out_im is 9 cycles and from the start to the done is 13 cycles but from the start to the all calculations end mem_re and mem_im being filled with the output is 12 cycles that because the done state take one cycle from the end of calculation to go to 1 since it’s sequential.

We can conclude that the values saved in mem_re , mem_im are the same as stored in the expected_data.

Latency=9 clock cycles.

Execution=12 clock cycles.

Reading output after done

SQNR eqn

  Higher SQNR → less quantization noise → fixed-point result is closer to floating-point.

  Typical scale:

- > 60 dB → excellent (error barely noticeable)

- 40–60 dB → acceptable for most DSP

- < 30 dB → large visible distortion

Coverage Report

Code coverage (bcst) branch , conditional , statement , toggle

fft_ctrl module

Why it’s not 100% because of the case default we can remove it since we covered all 4 cases

Why toggle don’t reach high coverage % ,because we used only one test case. For more we can run it 100 times for example.

twiddle_rom module

Also because of the case

reg_file module

butterfly module

fft8_top module

The total coverage is

ASIC phase

For the physical flow design we will use OpenLane opensource tool with technology and std cell used skywater 130nm , sky130_fd_sc_hd.

We will run flow.tcl and then the reports will created if the flow run successfully for all steps including synthesis and routing and power and pnr and cts …..

But we will do it later.

Thank

You